CLIM-715 Ground Flux Experiment

Everything you need to explain the experiment in one sitting.

Source: run_experiments.py + modified_full_report_1.docx. Built 2026-05-11.

1. The Big Picture (30-second version)

We simulate one-dimensional heat conduction through three urban substrate columns (asphalt road, concrete roof, bare soil), each 2 metres deep, driven by a synthetic diurnal cycle of solar radiation and air temperature. We solve the heat equation using three different finite-difference schemes (FTCS, BTCS, Crank-Nicolson) at three different time steps (15 s, 60 s, 600 s). The surface temperature is computed at every step by solving the surface energy balance with Newton iteration. We then measure how the schemes' surface ground heat flux deviates from a fine-Δt reference.

Headline finding: at Δt = 600 s, BTCS over-amplifies the diurnal G amplitude by 40 % on asphalt and concrete, 19 % on bare soil; Crank-Nicolson halves these errors. The dominant error is operator-splitting between the column and SEB updates.

2. Sequence of Steps — How the Experiment Runs

The whole project follows this order; everything else is detail.

Define substrates. Define three substrates as layered stacks of (depth, λ, C).

Build the grid. Build a stretched vertical grid for each substrate (top cell 0.5 cm for asphalt and concrete, 1 cm for bare soil; geometric stretch 1.18-1.25 down to bottom cell ≈ 30 cm at z = 2 m).

Initialize. Initialize substrate temperatures from the analytical damping-depth profile.

Time-step loop. Loop over time steps. For each step:

Sub-step A: advance the column from time n to n+1 using the θ-method, holding Tₛ⁰ fixed.

Sub-step B: solve the surface energy balance for new Tₛ⁰ via Newton iteration, holding the column fixed.

Apply bottom BC: T[N-1] = T[N-2] (zero-flux Neumann).

Integration period. Run for 2 diurnal cycles. Use day-2 (after 1-day spin-up) for all diagnostics.

Compare. Compare each scheme's day-2 surface ground heat flux G(t) and surface temperature Tₛ⁰(t) against the reference run (FTCS at Δt = 15 s).

Diagnose. Report: RMSE of Tₛ⁰, peak-amplitude ratio of G, daily-storage ratio.

3. The Three Substrates

Each substrate is a 2 m deep column of layered material with prescribed albedo at the top.

| Substrate | Layers (depth, λ W/m/K, C ×10⁶ J/m³/K) | Albedo α_s | d (cm) |
| --- | --- | --- | --- |
| Asphalt road (4 layers) | Asphalt 0-5 cm (0.75, 2.0); aggregate 5-25 cm (1.40, 2.4); dry soil 25-100 cm (0.30, 1.3); subsoil 100-200 cm (0.50, 1.8) | 0.10 | 10.2 |
| Concrete roof (3 layers) | Concrete deck 0-10 cm (1.50, 2.1); mineral-wool insulation 10-20 cm (0.04, 0.08); drywall/wood 20-200 cm (0.15, 1.5) | 0.30 | 14.0 |
| Bare soil (uniform) | Sandy loam 0-200 cm (0.30, 1.3) | 0.20 | 8.0 |

d = √(2κ/ω) is the diurnal damping depth, the e-folding depth of the daily wave. Computed from each substrate's top-layer κ = λ/C.

4. The Governing Equation (the physics)

The heat equation in conductivity form (preserves discrete heat conservation across layer interfaces):

C_s(z) · ∂T_s/∂t  =  ∂/∂z [ λ_s(z) · ∂T_s/∂z ]

In words: the volumetric heat capacity times the rate of temperature change at depth z equals the spatial divergence of the conductive heat flux. Heat piles up wherever more flux enters from above than exits below.

5. The Three Schemes — One Equation, Three α Values

All three time-stepping schemes are special cases of a single θ-method update equation:

C_j Δz_j (T_j^(n+1) − T_j^n) / Δt = α [−∂G/∂z]^(n+1) + (1−α) [−∂G/∂z]^n

where the flux-divergence at cell j is computed from G_{j±1/2} = λ_{j±1/2} (T_j − T_{j±1}) / (centre-to-centre spacing). Choose α; get a scheme:

| α | Scheme | Stability | Order in Δt | Per-step cost |
| --- | --- | --- | --- | --- |
| 0 | FTCS (Forward in Time) | Conditionally stable: ν ≤ 1/2 | 1st | Cheap (vectorised arithmetic) |
| 1/2 | Crank-Nicolson | Unconditionally stable | 2nd | Tridiagonal solve, O(N) |
| 1 | BTCS (Backward in Time) | Unconditionally stable, damping | 1st | Tridiagonal solve, O(N) |

ν = κ Δt / Δz² is the dimensionless diffusion number — the diffusion analogue of the Courant-Friedrichs-Lewy number.

6. Boundary Conditions — What Closes the System

The heat equation is second-order in z, so we need two BCs (one at each end of the column).

6a. Top BC — Surface Energy Balance (SEB) closure

The top of the column is closed by the requirement that the four surface fluxes balance to zero:

R_n − H − LE − G = 0   (solved for T_s⁰)

with each flux evaluated at the surface temperature T_s⁰:

Net radiation. R_n = (1 − α_s) S↓ + ε_s L↓ − ε_s σ (T_s⁰)⁴   (net radiation; σ = 5.67×10⁻⁸ W/m²/K⁴)

Sensible. H = ρ c_p (T_s⁰ − T_a) / r_a    where r_a = 1 / (C_H U) ≈ 67 s/m   (sensible heat flux)

Latent. LE = 0   (strict-impervious assumption — no evaporation)

Ground. G = λ_{1/2} (T_s⁰ − T_1) / (z_1 − z_0)   (ground heat flux, from the top half-level)

The (T_s⁰)⁴ term makes this equation nonlinear in T_s⁰. We solve it by Newton iteration:

T_s⁰_new = T_s⁰_old − F(T_s⁰_old) / F′(T_s⁰_old)

where F = R_n − H − LE − G. Typically converges in 3-4 iterations to |ΔT| < 10⁻⁴ K.

6b. Bottom BC — Zero-flux Neumann at z = 2 m

At the deepest face, the temperature gradient is set to zero:

∂T_s/∂z |_{z=2m} = 0    ⟺    G|_{z=2m} = 0    ⟺    T_{N-1} = T_{N-2}

Discretely, one line of code: the deepest cell mirrors the one above it. Justified because z = 2 m is 14-25 damping depths down, where the diurnal amplitude is already negligible (~10⁻⁶ of the surface value).

7. Von Neumann Stability Analysis — How We Used It

We used von Neumann analysis to PREDICT which (scheme, Δt, substrate) combinations would blow up, then VERIFIED the prediction empirically in Test 1 and Test 2.

7a. The procedure (four steps)

Substitute a Fourier mode  T_j^n = A^n · e^(ikjΔz)  into each scheme's update equation.

Cancel common factors and solve for the per-step amplification factor A as a function of ν and kΔz.

Demand |A| ≤ 1 for every resolvable wavenumber, i.e., kΔz ∈ [0, π].

Note that the worst case is always at kΔz = π (the 2-Δz wave, where adjacent cells flip sign). Substitute that and solve for the stability bound on ν.

7b. Results — the three amplification factors at kΔz = π

| Scheme | A at worst wave | ¦A¦ over ν > 0 | Stability bound |
| --- | --- | --- | --- |
| FTCS | 1 − 4ν | > 1 when ν > 1/2 | Conditional: ν ≤ 1/2 |
| BTCS | 1/(1 + 4ν) | Always in (0, 1] | Unconditional |
| Crank-Nicolson | (1 − 2ν)/(1 + 2ν) | Always in [−1, 1] | Unconditional |

7c. The 27-cell FTCS prediction (Test 2)

Plugging each substrate's top-cell κ, top-cell Δz, and the three Δt values into ν = κΔt/Δz² gives the FTCS stability picture in advance:

| Substrate (top κ, Δz₀) | ν at Δt=15s | ν at Δt=60s | ν at Δt=600s |
| --- | --- | --- | --- |
| Bare soil (2.31e-7 m²/s, 1.0 cm) | 0.035 ✓ | 0.139 ✓ | 1.39 ✗ (mild) |
| Asphalt road (3.75e-7 m²/s, 0.5 cm) | 0.225 ✓ | 0.900 ✗ | 9.00 ✗ (catastrophic) |
| Concrete roof (7.14e-7 m²/s, 0.5 cm) | 0.428 ⚠ | 1.71 ✗ | 17.14 ✗ (catastrophic) |

Test 2 then confirmed these predictions: FTCS blew up at Δt=60s on asphalt and concrete, at Δt=600s on all three substrates (mildly on soil). BTCS and CN completed every run.

8. The Two Experiments — Test 1 and Test 2

Test 1 — Verification (does the solver work?)

| Setting | Value |
| --- | --- |
| Substrate | Uniform sandy loam (1 material, no layers) |
| Grid | Uniform Δz = 1 cm everywhere |
| Surface BC | Prescribed: T_s⁰(t) = 292.5 + 7.5·cos(ω t) K |
| Bottom BC | Zero-flux Neumann at z = 2 m |
| Reference | Closed-form analytical damping-depth solution |
| Integration | 5 diurnal cycles; day-5 used for diagnostics |
| Configurations | 6: FTCS at ν=0.4 and ν=0.6; BTCS and CN at Δt=300 s and 900 s |
| Question answered | Does the solver reproduce the textbook analytical answer? (Yes.) |

FTCS at ν = 0.6 was predicted to blow up. It did, at step 53 — matching the predicted geometric growth factor |1 − 4·0.6|^53 ≈ 5.5×10⁷ per step. Confirmation of the theory.

Test 2 — Prognostic SEB (what we actually wanted to know)

| Setting | Value |
| --- | --- |
| Substrate | Three: asphalt road, concrete roof, bare soil |
| Grid | Stretched per-substrate: top 0.5-1 cm, geometric stretch 1.18-1.25, bottom ≈ 30 cm |
| Surface BC | Prognostic: T_s⁰ from SEB Newton iteration (top BC §6a above) |
| Bottom BC | Zero-flux Neumann at z = 2 m |
| Reference | FTCS at Δt = 15 s on the same substrate |
| Integration | 2 diurnal cycles; day-2 used for diagnostics |
| Configurations | 27: 3 substrates × 3 schemes (FTCS/BTCS/CN) × 3 Δt (15 / 60 / 600 s) |
| Question answered | How do FTCS, BTCS, CN compare at operational time steps on real urban substrates? |

9. Algorithm — What Runs Inside One Time Step

For each substrate × scheme α × Δt cell of the matrix, the following loop executes:

for step in range(N_steps):

# SUB-STEP A: column update (theta-method)

G_half = harmonic_mean_lambda * (T - T_neighbour) / dz_centre   # half-level fluxes

if alpha == 0:

# FTCS — explicit, no linear system

T_new = T + dt / (C * dz) * (G_half[:-1] - G_half[1:])

else:

# BTCS (alpha=1) or CN (alpha=0.5) — implicit, tridiagonal solve

A_band, b = build_tridiagonal_system(T, alpha, dt, dz, lambda_half, C)

T_new = scipy.linalg.solve_banded((1, 1), A_band, b)

# Apply bottom BC (zero-flux Neumann)

T_new[N-1] = T_new[N-2]

# SUB-STEP B: SEB update (Newton iteration for new T_s^0)

Ts0 = Ts0_old

for _ in range(MAX_ITER):

F  = R_n(Ts0) - H(Ts0) - LE - G(Ts0, T_new[1])

dF = dR_n/dT - dH/dT - dG/dT

dTs = -F / dF

Ts0 = Ts0 + dTs

if abs(dTs) < 1e-4: break

# Update top cell to the new surface temperature

T_new[0] = Ts0

# Advance time

T = T_new

t = t + dt

record(Ts0, G_half[0])

Order of operations is the key design choice: column FIRST (holding T_s⁰ fixed), then SEB SECOND (holding column fixed). This is the operator splitting that produces the first-order error analysed in §5.2 of the report.

10. What the Results Showed (one-page summary)

From Test 1 (verification)

FTCS at ν = 0.4: RMSE_T = 0.007 K — essentially exact.

FTCS at ν = 0.6: BLEW UP at step 53 — exactly as von Neumann predicted.

BTCS error grows linearly with Δt (first-order). CN error stays flat (second-order behaviour at this regime).

Conclusion: solver passes verification. Trust it on Test 2.

From Test 2 (prognostic SEB)

At Δt = 15 s: all three schemes agree to line-thickness on every substrate. Control condition.

At Δt = 60 s: FTCS blows up on asphalt (ν=0.90) and concrete (ν=1.71). BTCS and CN run to completion.

At Δt = 600 s: FTCS catastrophically unstable on asphalt and concrete; mildly unstable on soil. BTCS over-amplifies the diurnal G amplitude by 40 % / 41 % / 19 % (asphalt / roof / soil). CN halves these errors to 21 % / 26 % / 10 %.

Δt-refinement ratios: all six BTCS and CN values cluster in [8.2, 10.2] — first-order in Δt for both schemes, even though CN is intrinsically second-order. This is the smoking-gun signature of operator-splitting error.

SHAP attribution on a 150-column synthetic ensemble: κ_top is the dominant predictor of BTCS coarse-Δt error, with R² = 0.92 from κ_top alone.

Daily storage integral preserved within ±5 % across all schemes — so the over-amplification is symmetric (daytime overshoot + nighttime undershoot), preserving the daily mean. Implication: UHI diurnal range is biased, but UHI daily mean is preserved.

Why the operator-splitting error dominates

At every step, sub-step A advances the column with the OLD surface temperature, and sub-step B then updates the surface temperature with the NEW column. The column therefore equilibrates to the wrong T_s⁰ during sub-step A, and the discrepancy manifests as a spurious top-cell gradient. This error is first-order in Δt regardless of the per-substep scheme — which is why CN (intrinsically second-order) shows the same ratio of 10 as BTCS (first-order) when Δt scales by 10.

Two fixes were sketched in §5.4 of the report but not implemented: a fully coupled SEB row in the tridiagonal solve, or a Strang-symmetric splitting (A → B → A with half-step intervals). Either would eliminate the leading first-order splitting error.

Appendix — The atmospheric forcing functions

Synthetic, fully reproducible, sinusoidal:

S↓(t) = max[1000 · cos(ω(t − 12 h)), 0]  W/m²

L↓(t) = 350 + 20 · cos(ω(t − 14 h))     W/m²

T_a(t) = 292.5 + 7.5 · cos(ω(t − 14 h)) K

U(t) = 3 m/s   (constant)

with ω = 2π / 86400 s ≈ 7.27 × 10⁻⁵ rad/s (one full cycle per day). Solar peaks at noon; air temperature and longwave peak 2 hours later at 14:00 LT, modelling the typical afternoon thermal lag.

— End of cheat-sheet —